APPN-Hi-Res FIELDBOOK

NOTE FROM ARDEN: ADD PHOTOS IF YOU HAVE THEM, Or screen shots if possible.

ARDEN: Please DO NOT edit this anymore, I am moving it to the git repository.

This fieldbook provides a standardised operational guide for APPN Hi￼Res UAV deployments using the Phase One iXM￼GS120 120 MP RGB camera mounted on a DJI M350 RTK. It is intended for trained APPN staff conducting high￼resolution RGB data acquisition for crop emergence, canopy architecture, and structural phenotyping applications. The protocol supports safe flight operations, consistent sensor configuration, and high￼quality, repeatable data capture across APPN field nodes.

This protocol must be followed for all standard APPN Hi￼Res UAV flights. For any flights that fall outside standard operating procedures, detailed records must be kept documenting all deviations and their implications for data quality and downstream analysis.

EQUIPMENT

Ensure batteries for all equipment are fully charged before heading to the field. Ensure charging cables are available for necessary equipment.

Aircraft

  - DJI M350 (with P3 gimble damping plate attached)

  - Aircraft batteries and spares

  - Landing gear

  - Landing pad

  - Spare parts

  - Tools

  - Logbook

- Radio Control Transmitter / Ground Control Station

Hi-Res sensor payload

  - Phase One iXM￼GS120 camera body and P3 gimble

  - 80 mm lens

  - Camera storage media (CFexpress) and card reader

- Ground reference kit

  - DJI D-RTK 2 base station, tripod and spare batteries

  - Ground control points

  - GNSS system (Emlid / Trimble)

  - Wooden base station peg with marked centre-point (Optional – for when returning to location and conducting surveys through time, or needing to link to real-world GNSS data)

Accessories

  - Safety gear (signage and high vis vests)

  - Aeronautical radio

  - Field laptop and spare batteries

  - External storage media

  - Water, food, esky, sunscreen, bug spray, first aid kit, etc.

  - Spirit bubble, spirit level (or angle measurement) and measuring tape

  - External power brick (For charging UAV RC)

  - Handheld anemometer (such as a Kestral)

  - Professional lens cleaning wipes

FIRST USE AND PRESEASON PREPERATION

Performance of the camera relies on correct installation of the PhaseOne camera within the gimbal, a perfectly balanced gimbal, and the drone performs best when the camera weight and centre of gravity has been factored into the DJI flight control software.

Please refer to the “NON ROUTINE, FIRST TIME, AND COMMISSIONING PROCEDURES” instruction at the end of the document, for information on installing the damper plate, gimbal installation and setup, gimbal balancing and drone centre of gravity balancing.

NOTE: Once you have set the gimbal balance, note the settings and offsets and take photos of the setting locations to ensure that you have reference settings for a stable gimbal. If the camera and gimbal goes out of balance, immediately inspect the balance settings, and set back to you known default balanced values for that camera.

PREFLIGHT PLANNING – M350

Ensure that you apply for UAV flight approvals for locations and dates of flights well in advance.  Further planning documentation for the Phase One payload on the M350 can be found here. Note also that before first flight with this sensor, there are several steps that need to be completed that are documented at the end of this protocol.

- Using a GPS survey system (Emlid, Trimble…) or a GIS software create a polygon of the area of interest. Make sure your polygon includes the areas where you will place your GCPs, with an additional 5m buffer to avoid incomplete data.

- Create a polygon of the area of interest.

- Save the polygon as a KML file using the convention: YYYYMMDD_XXXX (where YYYY = year; MM = Month; DD = Day (must be 2 digits), and XXXX is a short name reference or abbreviation for the job).

- Load this KML file onto a USB stick or microSD card for transfer to M350 controller.

- Power on controller.

- Open DJI Pilot 2 and select Flight Route → + Import Route (KMZ/KML). Alternatively, you can create an Area Route on the controller (such as by walking the field). Note that you do not need the UAV powered on for this, just the controller.

- Confirm that the Phase One iXM-GS120 payload is correctly detected in DJI Pilot 2 and selected as the active payload for the mapping mission. Ensure the correct aircraft model (M350/M300 RTK) is selected, and configure a custom camera profile using the appropriate Phase One P3 (80mm) parameters.

- Set the flight altitude mode to Relative to Take￼off Point.

- Set image overlap and triggering parameters in the Advanced Settings of the mapping mission as follows:

- Front (forward) overlap: 80%

- Side (lateral) overlap: 80%

- Photo mode: Distance Interval Shot

- Set the course angle to align with the experimental layout:

- Use row aligned orientation for plot-based trials where crop rows are clearly defined.

- Use North-South orientation for broad scale paddock surveys.

- Set the flight altitude mode to Relative to Take￼off Point.

- The selected flight altitude and associated ground sampling distance (GSD), flight speed according to the mission standard type settings below. You must setect settings within the APPN HiRES Mission Standard Type Settings (Table 1), as these are optoimised to balance spatial resolution, coverage, and motion blur risk. The chosen altitude must be documented in the flight log.

Table 1 — APPN HiRES Mission Standard Type Settings.

Pre-tuned parameter bundles for the four APPN HiRES Mission Standard Types. Flight parameters must match the application and must not deviate from these settings. You can validate shutter speed and flight speed (to ensure no motion blur) against Table 2 (see below) for the specific speed you select.

| Standard Mission Type | Scenario | Altitude / Speed / Shutter / Overlap |
| --- | --- | --- |
| Type 1 | Plant counting / small structures (flowers, heads, early lesions, emergence counting) | Alt: 12–20 m · Speed: 2–3 m/s · Shutter: ≥1/4700 · Aperture: f/8 Overlap: 80/80 · |
| Type 2 | Plot phenotyping (canopy traits, vigour) | Alt: 30–50 m · Speed: 4–5 m/s · Shutter: ≥1/4700 · Aperture: f/8 Overlap: 80/80 |
| Type 3 | Canopy coverage / plot averages | Alt: 50–70 m · Speed: 5–7 m/s · Shutter: ≥1/4700 · Aperture: f/8 Overlap: 80/80 |
| Type 4 | Large-area flat terrain ortho (efficiency mode) | Alt: 80–120 m · Speed: 8–10 m/s · Shutter: ≥1/5200 · Aperture: f/8 Overlap: 80/80 |

- Disable Elevation Optimisation for standard agricultural surveys unless operating in highly variable terrain.

- Use the Phase One flight calculator to optimize flight speed whilst minimizing blur. This may need to be changed on site if lighting conditions require longer exposures.

- Press on the camera image icon to set the camera settings appropirate for your APPN HiRES Mission Standard Type.appropriatere to a fixed value of f/8.

- Set Shutter Speed according to Table 1. As a general rule, you should neber haev a shutter speed slower than 1/2000 sec (as this will result in motion blur).

- Set Focus to infinity.

- Set the ISO range (Min: 200 / Max: 3200). ISO is the only exposure value that should adjust automatically during the mission, depending on light conditions.

- Display the live histogram and confirm there is no sustained clipping in the highlights and shadows. You should not use any EV compensation (+/-), allow ISO to adjust automatically within the configured range.

- White Balance is fixed to Sunny / 5500K. (Apply this setting in all light conditions)

NOTE: for more information on flight settings and what they do, refer to Table 2 towards end of document(below)

- Before take-off, test-fire the camera (from the DJI Pilot 2 app and M350 Controller) and confirm that the image is written successfully to the storage media.

- Ensure any legacy data has been downloaded and backed-up. Format memory card from the DJI Pilot 2 app menus within the M350 Controller.

- Before starting the mission, manually fly to the planned operating altitude with the camera pointing down at the crop/plants. Confirm that the live preview and histogram show correct exposure.

- Save mission using the convention: YYYYMMDD_XXXX (where YYYY = year; MM = Month; DD = Day (must be 2 digits), and XXXX is a short name reference or abbreviation for the job).

ONSITE PREFLIGHT OPERATIONS – M350

- Conduct airspace and weather checks.

- Check NOTAM (find in ERSA).

- Ideally, no cloud cover. Avoid rapidly changing cloud conditions where possible.

- Maximum wind speed for quality capture is 6m/s or 22km/hr. Any windspeed over 5m/s or 18km/hr should be recorded.

- Do not operate in conditions below 0°C or above 40°C.

- Turn on aeronautical radio and set to local CTAF (find in ERSA).

- Set up DJI RTK 2 base station on tripod and power on at least 15 minutes prior to flight. Ensure it is set up in operation mode 5 (five LED blinks for communication with M350.

For any missions returning to the same location or requiring alignment to non-DJI data, the DJi D-RTK-2 base station must be set-up over a known point, that is surveyed in against a a GNSS base station(E.g. using AUSCORS or AUSPOS), or set to the location that was used for a previous survey. Enter into the M350 COntroller and RTK settings. Manually edit the latitude and longitude of the best station, to set to the known surveyed (or past) location. The Aircraft should be switched off during this step.

- Deploy GCPs in the field ensuring good coverage of the area of interest (see diagram below). If using Propellor Aeropoints, ensure they are switched on and logging. If using standard GCPs, log GNSS coordinates using a Trimble or Emlid GNSS system.

- Setup landing pad and UAV in a safe RTH location.

- In dusty environments an additional tarp should be used under the landing pad

- Attach Hi-Res payload to UAV:

- Check that the lens and sensors are clean. If not, use professional lens cleaning wipes to clean them.

- Check damping plate to ensure it is mounted securely to UAV and that USB-C cable is connected.

- Remove the protective metal cap from the quick release receptacle.

- Insert the gimbal into the quick release receptacle on the damping plate, while aligning the unlocked symbol on the bayonet opposite the white mark on the receptacle.

- Rotate the non-slip ring to the right until the locked symbol is opposite the white mark on the receptacle and the bayonet clicks and locks. Note: This mount is tool-less, but correct alignment is critical.

- Remove lens cap.

- Ensure CF express card is installed and that there is adequate space to store mission data.

- Power on radio controller; check battery status.

- Launch DJI Pilot 2.

- Load and review flight plan; checking operational height and double-checking area of interest, GCPs and reflectance panels are all within capture polygon.

- Power on aircraft; confirm connection to RC/GCS and battery status.

- On the controller, set up a safe UAV RTH location, RTH altitude and other geo-fencing settings. Check that mission is available on the controller.

- If you are using RTK, check that you are receiving GNSS corrections. There should be more than >8 satellites for good correction. Check the RTK resolution in the M350 controller settings, including that the solution is “FIX”, and the horizontal and vertical error is less than 0.03 (3cm).

FLIGHT OPERATIONS

- Ensure aircraft is in Position flight mode.

- Clear people/objects away from UAV.

- Notify crew/observers that takeoff is beginning.

- Begin manual takeoff, fly quickly to ~12m AGL and check stick controls work.

- Enable autonomous mission prepared earlier.

- Monitor UAV battery voltage/percentage in flight.

- After mission, switch back to Position mode to regain manual control.

- Land and disarm UAV.

- Leave UAV, transmitter, and sensor powered on; begin post-flight checks.

IN-FLIGHT DATA CONFIRMATION

- On RC Plus controller check image count and remaining storage on card (to ensure card did not fill up during mission). Review the images as the mission proceeds. Ensure that the image count is increasing as the images are written to the card. Listen for the “click” noise as images are captured. Review the histogram settings and the photo, to ensure that images have appropriate illumination.

POST-FLIGHT

- Power off drone then controller.

- Inspect drone for any damage and report for maintenance if necessary. Feel both batteries for excess or differential temperature. Feel the gimbal motors for excess or differential temperature. Inspect propellors, airframe and landing gear.

- Remove aircraft batteries.

- Remove payload, place in case.

- Pack up drone and all gear from site double checking against checklist at the start of this document to ensure everything is accounted for.

OFFLOAD DATA

Remove CFExpress card from camera and offload data to computer using card reader.

After offload and backup, format card for next mission.

GOBI SENSOR CONFIGURATION (Comprehensive)

Table 2 — Consolidated mission, airframe, camera and workflow settings. All values assume the  80 mm AF lens on the iXM-GS120 body; substituting any other lens or camera body invalidates the GSD column in Table 2 and requires re-calculation.

| Parameter | Recommended Setting | Operational Note |
| --- | --- | --- |
| Mission Type & Route Setup (DJI Pilot 2 — Mapping) |  |  |
| Mission Type | Area Route – Ortho Collection | Use the dedicated Ortho Collection template. Do not use Oblique Collection for orthomosaic output. |
| Ortho GSD (cm/pixel) | Leave default — auto-populates from Route Altitude | Altitude drives GSD via the Phase One sensor geometry. |
| Altitude Mode | Relative to Takeoff Point (ALT) | Assumes takeoff elevation ≈ site elevation. |
| Route Altitude (m) | Select from GSD Table (Table 2) — tailor to mission objective | Counting plant organs / flowers / heads → lowest practical altitude (12–20m). Canopy coverage / plot averages → 50–120 m. |
| Elevation Optimisation | DISABLED | Do not enable for ortho missions as it can causes alignment issues in Metashape. |
| Speed (m/s) | As fast as mission allows (recommended between 3 to 10m/s max) cross-check with Shutter Speed Lookup (Table 3) | Motion Blur = Shutter Interval (s) × Drone Speed (m/s). Rule: Motion Blur ≤ GSD. Example 1: 50 m altitude, 4 m/s, 1/4000 s. - Shutter interval: 1 ÷ 4000 = 0.00025 s - Drone speed: 4 m/s - Motion blur: 0.00025 × 4 = 0.001 m = 1.0 mm - GSD at 50 m: 2.1 mm = 0.0021 m - Check: 1.0 mm ≤ 2.1 mm ✓ Pass (blur is ~½ pixel — ideal) |
| Frontal & Side Overlap (%) | 80/80 (default, high-quality ortho) | 80/80 is the true-ortho setting - minimises lean, best for crop/plant-structure work |
| Route Start Point | Furthest polygon corner from pilot | Mission works back toward the pilot, finishing closest to the takeoff point. Reduces return-transit risk if battery runs low. |
| Takeoff Speed (m/s) | 10-15 | This is transit-only speed (takeoff point → first waypoint). Safe to run faster than capture speed because no images are triggered in transit. |
| 2. Phase One iXM-GS120 (80 mm AF Lens) — Camera Capture Settings |  |  |
| Exposure Mode | Manual (Fixed Aperture + Fixed Shutter + Auto ISO) | Lock shutter speeds (1/2000 or faster – see table below), set aperture to f/8, let ISO adjust. This shutter and aperture priority ensures motion-blur ceiling is protected first, radiometric quality second. |
| EV Compensation | 0 baseline — adjust to +1 in low-light / emergence conditions | +1 EV pushes the histogram right for more tonal information. Verify with live histogram. |
| ISO | Base 200 (sensor native) - max 3200 | iXM-GS120 native ISO range is 200–6400 (RGB). Noise becomes visibly detectable above ISO 3200. |
| Aperture | f/8 min (sweet spot) — f/11 max | Sharpness peaks ~f/8 across the frame; edge softness appears wide of f/5.6 and diffraction softens past f/11. |
| Shutter Speed | Set per Shutter Speed Lookup (Table 3) — from 1/2000 to 1/16,000 | Motion blur rule: shutter × speed ≤ GSD × 0.5 (half-pixel tolerance). See table below |
| Focus Mode | Manual focus locked to infinity (> 3 m) OR AF (single acquisition at mission start) | A calibrated fixed-focus 80 mm (infinity-only) achieves ±2 px RMSE vs. higher residuals on an uncalibrated AF. For research-critical work, consider requesting the fixed 80 mm (Phase One team recommendation) |
| Focus Bracketing | OFF | Bracketing multiplies frame count and breaks consistent GSD assumptions for ortho processing. |
| White Balance | N/A. It’s fixed to Daylight (5500K) | Fixed WB ensures radiometric consistency for post-processing |
| File Format | IIQ (RAW, 14-bit) | Expect ~100 MB per IIQ frame. |
| Environmental Capture Window |  |  |
| Cloud Cover (preferred) | Clear blue sky OR uniform high-altitude cover (cirrus / cirrostratus) | Both yield consistent illumination across the mission. Uniform altostratus is acceptable if genuinely even. |
| Cloud Cover (avoid) | Low-level broken / patchy cumulus | Hard shadows move across the AOI during capture → per-frame radiometric inconsistency. |
| Sun Angle and Capture Window | Ensure sun is ±20°of solar noon, approximately 2 hours before or after noon | Solar noon will depend upon time of year and latitude so please check here if unsure |
| Wind | ≤ manufacturer limit for M350 RTK (12 m/s sustained); prefer ≤ 8 m/s for photogrammetric quality | Gust-induced position jitter degrades overlap consistency |
| GCPs, Boresight & Calibration |  |  |
| Ground Control Points | Minimum 5 per site (4 corners + 1 centre) Surveyed with RTK/PPK receiver, ≤ 2 cm accuracy | Required for absolute accuracy validation. For repeat monitoring plots, install permanent GCP markers. |
| Cross-Runs over GCPs | INCLUDE — fly a perpendicular cross-strip over the GCP line after the main mission | Per Phase One ANZ guidance, cross-runs are valuable for boresight angle determination, especially after CoG calibration and with gimbal aligned to nadir. Adds ~5 min per mission. |
| Gimbal Calibration | Verify nadir alignment before each flight day | Even small gimbal offsets propagate into exterior orientation errors across a full mission. |

GSD vs Altitude Lookup

Table 3 — Ground Sampling Distance reference for the Phase One iXM-GS120 with RSM 80 mm AF lens. Derived from pixel pitch (3.45 μm) × altitude / focal length (80 mm); values match the Phase One published GSD table.

| Flight Altitude (m AGL) | GSD (mm/px) | GSD (cm/px) | Typical Use Case |
| --- | --- | --- | --- |
| 12 | 0.5 | 0.05 | Ultra-high detail — flower/head counting, disease lesions |
| 20 | 0.9 | 0.09 | Plant-organ ID, tillering, early-stage phenotyping |
| 30 | 1.3 | 0.13 | Plot-level phenotyping, canopy architecture |
| 50 | 2.1 | 0.21 | Standard orthomosaic — canopy coverage, NDVI-analog |
| 70 | ~3.0 | 0.3 | Large-area plot-average traits |
| 100 | ~4.3 | 0.43 | Site-scale mapping, large trial blocks |
| 120 | 5.2 | 0.52 | Wide-area ortho - Max altitude standard ops |

Shutter Speed Lookup

Table 4 — Minimum shutter speed (expressed as 1/x of a second) required to keep motion blur below half a pixel at each altitude/speed combination (research-grade). Read down (altitude) and across (ground speed) to find the floor (values rounded up to the nearest hundred).

| Spd → / Alt ↓ | 1 m/s | 2 m/s | 3 m/s | 4 m/s | 5 m/s | 6 m/s | 7 m/s | 8 m/s | 9 m/s | 10 m/s |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 m | 1/4700 | 1/9400 | 1/14000 | 1/18700 | 1/23300 | 1/28000 | 1/32600 | 1/37300 | 1/41900 | 1/46600 |
| 20 m | 1/2400 | 1/4700 | 1/7000 | 1/9400 | 1/11700 | 1/14000 | 1/16300 | 1/18700 | 1/21000 | 1/23300 |
| 30 m | 1/1600 | 1/3200 | 1/4700 | 1/6300 | 1/7800 | 1/9400 | 1/10900 | 1/12500 | 1/14000 | 1/15600 |
| 40 m | 1/1200 | 1/2400 | 1/3500 | 1/4700 | 1/5900 | 1/7000 | 1/8200 | 1/9400 | 1/10500 | 1/11700 |
| 50 m | 1/1000 | 1/1900 | 1/2800 | 1/3800 | 1/4700 | 1/5600 | 1/6600 | 1/7500 | 1/8400 | 1/9400 |
| 60 m | 1/800 | 1/1600 | 1/2400 | 1/3200 | 1/3900 | 1/4700 | 1/5500 | 1/6300 | 1/7000 | 1/7800 |
| 70 m | 1/700 | 1/1400 | 1/2000 | 1/2700 | 1/3400 | 1/4000 | 1/4700 | 1/5400 | 1/6000 | 1/6700 |
| 80 m | 1/600 | 1/1200 | 1/1800 | 1/2400 | 1/3000 | 1/3500 | 1/4100 | 1/4700 | 1/5300 | 1/5900 |
| 90 m | 1/600 | 1/1100 | 1/1600 | 1/2100 | 1/2600 | 1/3200 | 1/3700 | 1/4200 | 1/4700 | 1/5200 |
| 100 m | 1/500 | 1/1000 | 1/1400 | 1/1900 | 1/2400 | 1/2800 | 1/3300 | 1/3800 | 1/4200 | 1/4700 |

NON￼ROUTINE, FIRST￼TIME, AND COMMISSIONING PROCEDURES

Damping plate installation (more details here)

Also refer: https://youtu.be/hAbc81LmwXU?si=PChF7W1bFw2N6Xq9

The P3 gimble require a custom damping plate on the front of the M350 UAV.

Power down aircraft and remove batteries.

Place aircraft in protective cradle and invert.

Remove USB￼C connector screws.

Disconnect USB￼C connector.

Remove DJI factory damping plate.

Install Phase One P3 damping plate.

Apply Loctite 222 to all mounting screws.

Torque mounting screws to specification.

Reconnect USB￼C connector.

Torque USB￼C screws.

Return aircraft upright.

iXM camera configuration via iX Capture (more details here)

Insert storage media into camera.

Connect camera to PC via USB-C.

Power aircraft to supply camera power.

Open iX Capture.

Format storage media.

Set LINK type = DJI M300/M350.

Restart camera.

Confirm camera reconnects successfully.

- Gimbal Balancing

The balancing of the gimbal is critical to the performance of the camera, including risks from blurred images and gimbal motor overheating. The gimbal must be balanced, such that the camera remains in a neutral position it is placed in. Follow the following steps and instructional video on gimbal balancing

- https://youtu.be/T-foiXM-1tk?si=UCUcDNS9dvxEnD_t

X

Y

Z

P

Q

- Record and photography the settings.... (add in Table)

- Drone Centre of Gravity Calibration

- The center of gravity will shift when the aircraft’s payloads change. To ensure stable flight, it is required to recalibrate the aircraft’s center of gravity when a new payload is installed.

Calibrate in a windless environment. Make sure that the aircraft is hovering and there is a strong GNSS signal during calibration.

- Maintain visual line of sight of the aircraft and pay attention to flight safety.

- Refer to this video for more information: https://youtu.be/Jx60AdmacfA?si=psUrBgX_gJ9GZ0cg

Go to Flight Controller Settings in the DJI Pilot 2 app.

Tap Calibrate in the Center of Gravity Auto Calibration section.

The Aircraft Status Indicators will glow solid purple during calibration.

There will be a prompt in the app after calibration is completed.

- Set up custom camera in DJI Pilot 2